AI IN STRUCTURAL HEALTH MONITORING FOR PREDICTIVE MAINTENANCE & SAFETY ASSURANCE
EXECUTIVE PROGRESS SUMMARY: PRIOR MDP, CURRENT IDP, DUAL-PLATFORM DEPLOYMENT & PROCUREMENT PLAN
BACSE291 — Innovative Design Project | Fall 2026 | Vellore Institute of Technology (VIT), Vellore
Guide: Dr. Senthil Kumar N (Emp ID: 15377, SCOPE) | Aerial (15377IDP01): Priyam S., Ayushman K. | Ground (15377IDP02): Nisargkumar J. (Lead), Aryan M., Mudit G.

| 1. PREVIOUS MDP: PLY FORGE POINT CLOUD PROCESSOR | 2. CURRENT IDP WORK DEVELOPED & VERIFIED |
| --- | --- |
| • Architecture: Client-side WebGL 1.0 3D point cloud processor in HTML5/ES6 without external libraries (no Three.js, no Node.js backend). • Core Capabilities: Binary/ASCII PLY parser (DataView/ArrayBuffer), 18,000-pt Stanford Bunny demo, circle ROI filtering, 2.5D Delaunay/grid meshing, and PLY export. • Voxel Downsampling: Spatial hash map downsampling point clouds by 60%–90% while preserving structural boundaries. • Direct IDP Role: Directly embedded into Ground Station Tab 4 (Streamlit WebGL Viewer) for zero-latency 3D facade rendering without external software. | • Aerial Sensing (15377IDP01): Calibrated Benewake TF-Luna 850nm dToF (<5g, 0.35W, 100 Hz) on ESP32 FreeRTOS. UART sync to 0x59 0x59 with checksum and median filter (N=5). • Empirical Metrology Fit: Universal model: y = 1.0228x + 3.20 cm (R²=1.0000, RMSE=±0.02 cm across 39.0cm and 61.4cm physical benchmarks). • Ground Analytics (15377IDP02): Rolling baseline tracking calculating residuals (Δd); flags spalls (Δd ≥ +3.5cm) and bulges (Δd ≤ -3.5cm). • 3D Digital Twin: Reconstructed 9,969-point room (.PLY) isolating Almari bulge (+65cm) and Gate recess (-60cm). 5-tab Streamlit dashboard. |

| 3. PLATFORM SPECIFICATION: DRONE VS. SMART 4WD CAR | 4. FUTURE ROADMAP & SYSTEM INTEGRATION |
| --- | --- |
| • Platform Duality: (1) Autonomous F450 Drone: Elevated facade inspection; (2) Smart 4WD Ground Car: Indoor corridors, eliminating prop-wash suction & hand jitter. • If Drone Used: Requires F450 airframe, 4x motors/ESCs, 3S LiPo, Pixhawk autopilot (+INR 17,350) -> Drone Total = INR 26,124.00. • If Car Used: Requires 4WD chassis, 1:48 gear motors, dual optical encoders, L298N driver, mast (+INR 3,830) -> Car Total = INR 12,604.00 (MVP: INR 9,964). • Shared Hardware: YDLIDAR X2 (INR 7,189) + Tilt Sensor (INR 245) + ESP32 (INR 380) + Regulators/Harness (INR 960) = INR 8,774.00. | • Phase 1–2 (Weeks 1–3): Procure Tier 1 hardware (LiDAR, tilt sensor, 4WD car kit); verify UART framing and 5V rail regulation. • Phase 3–4 (Weeks 4–6): Assemble F450 airframe & propulsion; implement silicone vibration-damping deck (>18 dB attenuation); benchmark hover thrust. • Phase 5–6 (Weeks 7–10): Laboratory wall scanning with Smart 4WD car (encoder odometry); tethered facade flights; blind defect detection. • Phase 7 (Weeks 11–14): End-to-end integration, Streamlit PLY Forge 3D meshing, automated compliance PDF reports, and IEEE submission. • Target Metrics: Static distance RMSE ≤ ±1.5cm; Tilt residual ≤ 0.5cm; Defect sensitivity ≥ 95%; Car odometry drift ≤ 1.0cm / 5m. |

| Hardware Package | Specific Inclusions & Role | Subtotal (INR) | When Required |
| --- | --- | --- | --- |
| 1. Shared Core Sensing | YDLIDAR X2 360° LiDAR (INR 7,189), DFRobot Tilt (INR 245), ESP32 MCU (INR 380), Buck/Harness (INR 960) | INR 8,774.00 | Required for BOTH Drone & Car |
| 2. Drone Platform Only | F450 Frame, 4x Motors/ESCs (INR 5,200), 3S 4200mAh LiPo & B6 Charger (INR 4,430), Pixhawk Autopilot (INR 5,950), Reserve | INR 17,350.00 | Required ONLY if Drone is Used |
| 3. Smart 4WD Car Only | 4WD Chassis + 4x Gear Motors (INR 1,050), Dual Optical Encoders (INR 320), L298N (INR 380), Mast (INR 650), 12V Li-ion (INR 1,150) | INR 3,830.00 | Required ONLY if Car is Used |
| TOTAL DUAL-PLATFORM | Complete All-Environment System: Shared Core + Drone Platform + Smart 4WD Car Platform | INR 28,874.00 | Car-Only MVP: INR 9,964.00 |