AI IN STRUCTURAL HEALTH MONITORING FOR PREDICTIVE MAINTENANCE & SAFETY ASSURANCE
EXECUTIVE PROGRESS SUMMARY: PREVIOUS MDP, CURRENT IDP, HARDWARE UPGRADE & PROCUREMENT PLAN
BACSE291 — Innovative Design Project | Fall 2026 | Vellore Institute of Technology (VIT), Vellore
Guide: Dr. Senthil Kumar N (Emp ID: 15377, SCOPE) | Aerial: Priyam S., Ayushman K. | Ground: Nisargkumar J. (Lead), Aryan M., Mudit G.

| 1. PREVIOUS MDP: PLY FORGE (POINT CLOUD PROCESSOR) • Client-side WebGL 3D Point Cloud Processor built in HTML5/ES6 without external libraries. • Verified features: Binary/ASCII PLY parsing, circle ROI filtering, 2.5D Delaunay/grid surface triangulation, and PLY export. • Voxel-grid downsampling achieving 60%–85% file compression while preserving structural boundaries. • Cross-Project Reuse: Directly powers the ground analytics 3D WebGL digital twin rendering engine, eliminating software licensing costs. | 2. CURRENT IDP WORK DEVELOPED & VERIFIED • Aerial Subsystem (15377IDP01): Benewake TF-Luna 850nm ToF (<5g, 0.35W, 100 Hz) interfaced with ESP32. Firmware parses 9-byte packets, validates 0x59 0x59 sync and 8-bit checksum. Running median filter (N=5). • Two-Point Calibration: Universal straight-line model: y = 1.0228x + 3.20 cm (R²=1.0000, RMSE=±0.02 cm on physical benchmarks). • Ground Subsystem (15377IDP02): Rolling baseline residuals (Δd = d_meas - d_ref); flags spalling cavities (Δd ≥ +3.5cm) and bulges (Δd ≤ -3.5cm). • 3D Digital Twin: Reconstructed 9,969-point room (.PLY) isolating Almari bulge (+65cm, Orange) and Gate recess (-60cm, Green). 5-tab Streamlit dashboard active. |
| --- | --- |
| 3. HARDWARE UPGRADES & PROCUREMENT BUDGET • Engineering Need: Manual sliding jitter (~2–3 cm) and sensor tilt elongation (d_true = d_meas · cos θ) require dedicated sensing and airframe mobility. • YDLIDAR X2 360° LiDAR: 3,000 pts/s, 7 Hz continuous planar slicing, 0.12–8.0m range, 3.3V UART (115200 bps). Ref price: ₹7,189.00. • DFRobot Gravity Tilt Sensor: 360° ball switch detecting pitch/roll to apply trigonometric compensation in ESP32 firmware. Ref price: ₹245.00. • F450 Drone Kit + Pixhawk: F450 frame, 4x 2212 motors, 4x 30A ESCs, Pixhawk 2.4.8 autopilot, 1045 props. Subtotal: ₹12,020.00. • Power & Mounting: 3S 4200mAh 35C LiPo (₹2,450), iMAX B6 charger (₹1,350), 5V 5A UBEC (₹380), silicone damper & wiring (₹740). • TOTAL BUDGET: Essential: ₹24,724 ¦ Shipping: ₹400 ¦ Contingency (8%): ₹2,000 ¦ TOTAL: ₹27,124.00 INR (MVP Option: ₹8,214.00). | 4. FUTURE ROADMAP & SYSTEM INTEGRATION • Phase 1–2 (Weeks 1–3): Procure Priority 1 hardware (LiDAR, tilt sensor, UBEC); verify UART framing and 5V rail brownout protection on bench. • Phase 3–4 (Weeks 4–6): Implement firmware tilt compensation (d_true = d_cal · cos θ); assemble F450 quadcopter airframe; balance center of gravity; install silicone vibration isolation deck (>18 dB attenuation). • Phase 5–6 (Weeks 7–9): Adapt PLY Forge WebGL engine for streaming UDP point cloud buffers; conduct tethered hover flight trials; scan facade with calibrated 2cm & 3.5cm simulated spalls. • Phase 7 (Week 10): End-to-end faculty demonstration, automated PDF inspection report generation, and longitudinal deterioration baseline establishment. • Target Metrics: Static distance MAE ≤ ±0.5cm; Tilt error residual ≤ ±1.0cm; Defect detection precision ≥ 90%, recall ≥ 85%; Continuous hover flight time ≥ 14 mins. |

| Component Group | Key Inclusions | Cost (INR) | Procurement Phasing |
| --- | --- | --- | --- |
| Sensors & Power Regulation | YDLIDAR X2 360° LiDAR (₹7,189), DFRobot Tilt Sensor (₹245), 5V 5A UBEC (₹380) | ₹7,814.00 | Priority 1 (Immediate / Week 1–2) |
| Airframe & Power Reservoir | F450 Frame Kit + Motors + ESCs (₹5,200), 3S 4200mAh LiPo & B6 Charger (₹3,800), Dampers (₹740) | ₹9,740.00 | Priority 2 (Integration / Week 3–4) |
| Avionics & Contingency | Pixhawk 2.4.8 Autopilot + GPS Kit (₹6,850), Courier Shipping (₹400), Risk Reserve (₹2,000) | ₹9,250.00 | Priority 3 (Flight Trials / Week 5–6) |
| TOTAL RECOMMENDED GRANT | Complete Airborne Cyber-Physical SHM System (MVP Bench Option: ₹8,214.00) | ₹27,124.00 | Faculty Sign-Off Ready |

Project Leads: Nisargkumar Joshi (25BCE0594, Ground Lead) & Priyam Sharma (25BAI0159, Aerial Lead) | Guide Sign-off: Dr. Senthil Kumar N (Associate Professor Grade 2, SCOPE) | Date: October 24, 2026