PROJECT TECHNICAL SPECIFICATION & ARCHITECTURAL REPORT

AI-Driven Robotic Structural Health Monitoring (SHM) & 3D LiDAR Metrology Suite

| DOCUMENT ID: TF_LUNA_PROJECT_DOCUMENT_R0 | REVISION: R0 (Revision Zero - Baseline) | DATE: October 2026 |
| --- | --- | --- |
| DOMAIN: Robotics, IoT & Metrology | PRIMARY SENSOR: Benewake TF-Luna (850nm ToF) | TARGET PLATFORM: ESP-32 MCU & Three.js WebGL |

1. Executive Summary

This Revision Zero (R0) technical document formally documents the system concept, sensor selection rationale, mathematical calibration, coordinate transformation methodology, and architectural digital twin pipeline for our AI-Driven Robotic Structural Health Monitoring (SHM) & 3D LiDAR Metrology Suite.

Civil infrastructure inspection and interior architectural layout verification have historically relied on manual physical tape measures and qualitative visual inspection. These legacy methods suffer from human error, sagging tape lines over long distances, high labor costs, and safety hazards in hard-to-reach locations. Our project introduces a lightweight, low-cost autonomous sensing framework that captures millimeter-accurate physical depth at up to 100 Hz, extrudes complete 3D watertight rectangular room twins, automatically classifies architectural protrusions and recesses (e.g. Almari vs. Gate), and renders interactive 3D WebGL point clouds.

2. Sensor Selection: Current (1D) vs. Upcoming (2D 360°)

2.1 Current Hardware: Benewake TF-Luna 850nm ToF Micro-LiDAR

For the initial phase (Revision Zero), the primary sensing payload is the Benewake TF-Luna Time-of-Flight (ToF) Micro-LiDAR sensor, directly interfaced with an ESP-32 microcontroller via UART (115200 baud).

Operating Principle: Measures distance using the pulse Time-of-Flight equation: d = (c * Δt) / 2, where c is the speed of light and Δt is round-trip time.

Wavelength: 850 nm Near-Infrared (NIR) Class 1 eye-safe VCSEL laser transmitter.

Operating Range: 0.20 m to 8.00 m (effective range on indoor walls and standard targets).

Optical Blind Zone: 0 to 20 cm. Due to physical optical parallax between the transmitter lens and receiver photodiode, objects closer than 20 cm do not focus properly onto the sensor array. Handled via automated blind-zone warnings.

Sampling Frequency: Up to 250 Hz (configured at 60-65 Hz in firmware for smooth USB communication).

Mass & Dimensions: Less than 5 grams (35 mm x 21.2 mm x 13.5 mm). Ultra-lightweight for micro-drones.

Power Consumption: 5V DC, operating current <= 70 mA (power consumption < 0.35 Watts).

Telemetry Interface: UART 115200 Baud, streaming standard 9-byte binary packets with checksum validation.

Rationale for Choosing TF-Luna:The TF-Luna was chosen for R0 because of its negligible weight (<5g), ultra-low power consumption (<350mW), high measurement rate (up to 250 Hz), and minimal unit cost (<$25). It allows rapid prototyping on micro-aerial platforms where every gram of payload weight directly impacts battery flight duration.

2.2 Active Calibration Model (Sub-Centimeter Precision)

Raw integer output from the TF-Luna contains a static optical zero-point housing offset (+3.20 cm) and minor distance-dependent scaling. Through dual physical benchmarks against certified steel measuring tapes at 39.0 cm and 61.4 cm, an exact 2-point linear regression model was established:

📌 TF-LUNA ON-CHIP MATHEMATICAL CALIBRATION MODEL Formula: Calibrated_Distance (cm) = (1.0228 * Raw_Distance) + 3.20 cm • Correlation Factor: R² = 1.0000 (Exact 0.00 cm residual error at physical calibration marks) • Sensor Stability: 8-sample continuous ring-buffer oversampling executed on ESP-32 firmware • Reference Plane: Front glass face of the TF-Luna optical lenses represents the 0.00 cm mark.

2.3 Upcoming Sensor: 2D 360° Planar LiDAR (Phase 2 / Revision 1)

For the next engineering milestone, the system will integrate a 2D 360° Rotating Laser Range Scanner (e.g., RPLiDAR A1/A2 or LD06 / YDLidar series) alongside the TF-Luna.

Scanning Mechanism: Rotating laser transceiver head spinning at 5.5 Hz to 12 Hz (330 to 720 RPM).

Field of View (FoV): Full 360° continuous omnidirectional horizontal plane coverage.

Ranging Range: 0.15 m to 12.00 m.

Sampling Speed: 2,000 to 8,000 samples per second.

Coordinate Generation: Converts polar tuples (r, θ) directly into Cartesian coordinates: X = r*cos(θ), Z = r*sin(θ).

Engineering Rationale for 2D LiDAR:The transition to a 2D 360° LiDAR provides significant advantages: (1) Instantaneous whole-room perimeter capture in a single 100ms sweep; (2) Elimination of manual hand-sweep speed variations; (3) Native support for Simultaneous Localization and Mapping (2D SLAM); and (4) Autonomous wall boundary closure.

2.4 Comprehensive Sensor Comparison Matrix

| Metric / Specification | Current Sensor: TF-Luna (1D) | Upcoming Sensor: 2D 360° LiDAR |
| --- | --- | --- |
| Dimensionality | 1D Single-Point Vector | 2D Omnidirectional Planar Slice |
| Field of View (FoV) | 2.0° Narrow Collimated Spot | 360.0° Continuous Circular Ring |
| Measurement Range | 0.20 m to 8.00 m | 0.15 m to 12.00 m |
| Sampling Rate | Up to 250 Hz (60 Hz paced) | 2,000 to 8,000 points/sec |
| Sensor Weight (Mass) | < 5 grams (Ultra-light) | 170 to 200 grams |
| Power Draw | < 0.35 W (70 mA @ 5V) | 1.5 W to 2.5 W (300-500 mA @ 5V) |
| Unit Cost | < $25 USD | $80 to $180 USD |
| Internal Motors | Zero (100% Solid-state) | Brushless motor + optical encoder |
| Scanning Method | Sequential 4-wall manual sweep | Instantaneous 360° single-revolution sweep |
| Typical Point Count | 1,500 to 10,000 pts per room | 50,000 to 200,000 pts per room |
| Best Deployment Fit | Micro-drones & payload stand-off hold | Ground rovers & autonomous SLAM mapping |

3. Coordinate Frame & Dynamic Translation Tracking

3.1 The Origin Datum (0, 0, 0) & Dynamic Displacement Principle

A core guideline highlighted during project evaluation is the mathematical modeling of sensor movement. When dragging, translating, or flying the sensor from an initial location, the coordinate system must not treat every measurement as originating from a stationary center. Instead, the sensor's own spatial position must be continuously tracked.

📌 DYNAMIC COORDINATE FRAME & SENSOR TRANSLATION MODEL 1. Initial Condition: At time t = 0, the sensor datum position is set to the absolute origin: P_sensor(0) = (X₀, Y₀, Z₀) = (0.000, 0.000, 0.000) 2. Dynamic Translation: As the sensor is dragged or moved across space along physical axes: P_sensor(t) = P_sensor(0) + ΔT(t) = (0 + ΔX(t), 0 + ΔY(t), 0 + ΔZ(t)) 3. Target Point World Coordinates: For any distance reading d(t) at angles (θ, φ): X_world(t) = ΔX(t) + d(t) * sin(θ) * cos(φ) Y_world(t) = ΔY(t) + d(t) * sin(φ) Z_world(t) = ΔZ(t) + d(t) * cos(θ) * cos(φ) Significance: When you drag the sensor, the measured coordinates dynamically update relative to the movement path, preventing point cloud collapse and preserving geometric realism.

Physical Meaning of Axes in the Room Twin Frame:

X-Axis: Lateral axis across the room (Width). Scanning left-to-right increments or decrements X.

Y-Axis: Vertical elevation axis (Height). Height extrusion from floor (Y = 0) to ceiling (Y = 2.40m).

Z-Axis: Longitudinal depth axis (Depth). Spans between North Wall (Z = +D_N) and South Wall (Z = -D_S).

4. 3D Digital Twin Architecture & Structural Feature Classification

4.1 Sequential 4-Wall Inspection Workflow

To generate a clean rectangular digital twin, the system executes a structured 4-wall sweep:

1. Wall 1 (North): Laser sweeps left-to-right across the front wall at Z = +2.40m (4.8m width).

2. Wall 2 (East): Laser sweeps left-to-right across the right wall at X = +2.40m (4.8m depth).

3. Wall 3 (South): Laser sweeps left-to-right across the back wall at Z = -2.40m (4.8m width).

4. Wall 4 (West): Laser sweeps left-to-right across the left wall at X = -2.40m (4.8m depth).

4.2 Three Structural Classification Categories

The algorithm evaluates deviation from the orthogonal wall baseline (Δd = d_measured * cos(θ) - d_baseline):

1. Normal Flat Wall (🟦 Cyan): Planar baseline boundary (|Δd| < 15 cm). Rendered in Cyan (#00E5FF, RGB: 0, 229, 255) forming the 4.8m x 4.8m room box.

2. Almari / Furniture Inward Bulge (🟧 Orange): Objects protruding into the room (Δd <= -15 cm), such as wardrobes, tables, or bookshelves. Rendered in Warm Amber/Orange (#FF8C00, RGB: 255, 140, 0). The 3D model extrudes an inward rectangular box (e.g., 'Almari: 65 cm Bahar') with an orange dimension badge.

3. Gate / Doorway Outward Recess (🟩 Green): Doorways, open gates, or alcoves recessed past the nominal wall line (Δd >= +15 cm). Rendered in Emerald Green (#00E676, RGB: 0, 230, 118). Extrudes an outward recessed box (e.g., 'Gate: 60 cm Andar') with a green dimension badge.

5. Software Implementation & Output Artifacts

The complete software stack comprises:

serial_reader.py: Reads 115200 baud serial packets, applies calibration formula, and logs to distance_data.csv.

continuous_multi_point_scanner.py: Executes 4-wall continuous sweep, performs rolling median noise reduction, extrudes 3D coordinates, and outputs reports/continuous_scan.ply.

reports/real_scan_3d_viewer.html: Standalone Three.js WebGL viewer with OrbitControls, dimension banners, and toggleable badges.

dashboard/app.py: 5-tab Streamlit web application with real-time SHM defect alerts, metrology tables, and 3D PLY model viewer.

data/room_mesh.ply & .obj: Standard 3D CAD/AR point clouds and wavefront mesh models (9,969 vertices).

6. Project Roadmap & Team Roles

| Member Name | Core Responsibilities | Revision Focus |
| --- | --- | --- |
| Nisarg Joshi | LiDAR Sensor Integration, Mathematical Calibration & 3D WebGL Engine | R0 Lead (Active) |
| Team Member 2 | Hardware Scanning Mechanism, Turret Mount & Power Management | R0 / R1 Hardware |
| Team Member 3 | Real-Time Telemetry Pipeline, Serial Drivers & Full-Stack Web Platform | R0 / R1 Telemetry |
| Mudit | AI Defect Segmentation, Point Cloud Classification & Spatial Clustering | R1 AI Engine |
| Aryan | Drone Telemetry, ESP32-CAM Wireless Interface & Flight Controller Integration | R1 Flight Payload |

Document Version: TF_LUNA_PROJECT_DOCUMENT_R0 | Status: Official Revision Zero Release